# 功能仿真计划

本文档说明 AXI Safety Island 功能仿真的测试平台、用例划分及复现方法。

# 1. 验证平台

| 项目 | 说明 |
| --- | --- |
| 仿真器 | Synopsys VCS T-2022.06+ |
| Testbench | tb/functional/tb_safety_island_top_full.v |
| DUT 顶层 | safety_island_top |
| 激励模型 | TB 内嵌 AXI Slave 存储器模型 |

# 2. 测试用例 (34 场景)

1. basic_fault_flow

2. no_fault_flow

3. multi_master_flow

4. burst_16_incr_flow

5. wrap_burst_flow

6. bus_error_flow

7. timeout_flow

8. outstanding_flow

9. out_of_order_flow

10. interleaving_flow

11. out_of_order_interleaving_flow

12. invalid_rid_error_flow

13. aou_rcheck_ok_flow

14. aou_rcheck_error_flow

15. aou_rcheck_burst_error_flow

16. config_error_flow

17. latent_fault_flow

18. expected_mismatch_flow

19. expected_match_flow

20. expected_masked_match_flow

21. heartbeat_pass_flow

22. heartbeat_fail_flow

23. heartbeat_no_interfere_flow

24. kat_pass_flow

25. kat_fail_flow

26. kat_disabled_flow

27. kat_araddr_corrupt_flow

28. tmr_state_ok_flow

29. tmr_state_minority_flow

30. tmr_state_double_fault_flow

31. tmr_fd_stuck_flow

32. tmr_cfg_locked_fault_flow

33. write_verify_pass_flow

34. write_verify_fail_flow

# 3. 赛题基本功能覆盖

| 测试项 | 对应用例 |
| --- | --- |
| AXI Slave 配置写入 | basic_fault_flow, config_error_flow |
| 5 路 Master 读取 | multi_master_flow |
| 超时机制 | timeout_flow |
| OK/Error Response | bus_error_flow |
| INCR/WRAP Burst | burst_16_incr_flow, wrap_burst_flow |
| Outstanding | outstanding_flow |
| Out-of-Order (附加) | out_of_order_flow |
| Interleaving (附加) | interleaving_flow |
| Mask+OR / fault_detect | basic_fault_flow, no_fault_flow |
| AoU 校验位 | aou_rcheck_*_flow |
| 配置保护 | config_error_flow, write_verify_* |
| Latent (附加) | latent_fault_flow |

# 4. 代码行覆盖率

功能仿真同时收集 Line/Toggle/Condition/FSM 覆盖率，结果输出至 sim/functional/coverage/。

| 模块 | Line % |
| --- | --- |
| 整体 | 87.5 |
| safety_island_core_logic | 86.7 (FSM 100%) |
| safety_island_fault_detector | 91.3 |
| tmr_voter | 100 |

# 5. 运行方法

cd submission/scripts && bash run_functional.sh

日志: sim/functional/logs/full_run.log